Current Date: 10/31/2022 7:25:03 AM

Units System: kip_in

Members

Actual members

Right brace	:	No

Left brace	:	Yes

Column

General

Section	:	W14X68

Material	:	STEEL

Column orientation	:	Transversal

Transversal offset	:	0 in

Include opposite stiffener	:	No

Left brace

General

Section	:	W12X26(T)

Material	:	STEEL

Brace slope angle (degrees)	:	49.399

Brace rotation (degrees)	:	90°

Double brace	:	No

Additional geometric data

wpx:  WP horizontal displacement	:	0 in

wpy:  WP vertical displacement	:	0 in

Le: Minimum distance to other members	:	7 in

L1: Left distance	:	0.5 in

L2: Right distance	:	0.5 in

Base plate

Base plate

Position on the support	:	Center

N: Longitudinal dimension	:	18 in

B: Transversal dimension	:	18 in

Thickness	:	1.5 in

Material	:	A572 Gr50

Column weld	:	E70XX

Outer welds flanges only	:	No

D: Column weld size (1/16 in)	:	6

Override A2/A1 ratio	:	No

Include shear lug	:	No

Support

With pedestal	:	Yes

Longitudinal dimension (pedestal)	:	36 in

Transversal dimension (pedestal)	:	36 in

Thickness	:	48 in

Material	:	C 4-60

Include grouting	:	Yes

Grout thickness	:	1.5 in

Anchor

Anchor position	:	Longitudinal position

Rows number per side	:	1

Anchors per row	:	2

Longitudinal edge distance on the plate	:	5.5 in

Transverse edge distance on the plate	:	5.5 in

Anchor type	:	Headed

Head type	:	Hexagonal

Include lock nut	:	No

Anchor	:	1 1/2"

Effective embedment depth	:	20 in

Total length	:	24.98 in

Material	:	F1554 Gr36

Fy	:	36 kip/in2

Fu	:	58 kip/in2

Cracked concrete	:	Yes

Brittle steel	:	No

Anchors welded to base plate	:	No

Anchor reinforcement

Type of reinforcement	:	Primary

Tension reinforcement	:	Yes

Tension bar size	:	no. 6

Tension bar grade	:	60 kip/in2

Tension number of bars	:	12

Shear reinforcement	:	Yes

Shear bar size	:	no. 4

Shear bar grade	:	60 kip/in2

Shear number of bars in major axis direction	:	4

Shear number of bars in minor axis direction	:	4

Interfaces

Left brace - Gusset

General

tp:  Thickness	:	0.375 in

Material	:	A572 Gr50

LV: Length on column	:	15 in

LH: Length over plate	:	8 in

Left brace - Gusset/Brace connection

General

Connection type	:	Bolted

Bolts	:	3/4" A325 N

Hole type	:	Standard (STD)

Hole type on gusset	:	Standard (STD)

nc:  Bolt columns	:	2

nr:  Rows of Bolts	:	2

s:  Pitch - longitudinal center-to-center spacing	:	3 in

g:  Gage - transverse center-to-center spacing	:	3 in

Leh:  Longitudinal distance to edge	:	1.5 in

Left brace - Gusset/Column connection

General

Connection type to column	:	Directly welded

Directly welded

Column weld	:	E70XX

D:  Weld size to column (1/16 in)	:	5

Left brace - Gusset/Base plate connection

Directly welded

Base plate weld	:	E70XX

D: Base plate weld size (1/16 in)	:	5