Current Date: 10/31/2022 7:25:44 AM

Units System: kip_in

Design code: AISC 360-16 LRFD, ACI 318-11

Interface between Gusset - Top left brace

Connection: Directly bolted

DEMANDS

Description	Pu	Load type

kip

LC-2053	27.37	Design

GEOMETRIC CONSIDERATIONS

Dimensions	Unit	Value	Min.	Max.	Sta.	References

Directly bolted

Transverse edge distance	[in]	1.74	1.00	-	    	Tables J3.4, J3.5

Lemin = edmin + C2 = 1 [in]  + 0 [in]  = 1 [in] 	Tables J3.4, J3.5

Longitudinal edge distance	[in]	1.50	1.00	-	    	Tables J3.4, J3.5

Lemin = edmin + C2 = 1 [in]  + 0 [in]  = 1 [in] 	Tables J3.4, J3.5

Transverse center-to-center spacing (gage)	[in]	3.00	2.00	9.12	    	Sec. J3.3, Sec. J3.5

smin = 8/3*d = 8/3*0.75 [in]  = 2 [in] 	Sec. J3.3

IsCorrosionConsideredFalse

smax = min(24*tp, 12  [in] ) = min(24*0.38 [in] , 12  [in] ) = 9.12 [in] 	Sec. J3.5

Longitudinal center-to-center spacing (pitch)	[in]	3.00	2.00	9.12	    	Sec. J3.3, Sec. J3.5

smin = 8/3*d = 8/3*0.75 [in]  = 2 [in] 	Sec. J3.3

IsCorrosionConsideredFalse

smax = min(24*tp, 12  [in] ) = min(24*0.38 [in] , 12  [in] ) = 9.12 [in] 	Sec. J3.5

Gusset

Transverse edge distance	[in]	2.44	1.00	-	    	Tables J3.4, J3.5

Lemin = edmin + C2 = 1 [in]  + 0 [in]  = 1 [in] 	Tables J3.4, J3.5

Longitudinal edge distance	[in]	1.50	1.00	-	    	Tables J3.4, J3.5

Lemin = edmin + C2 = 1 [in]  + 0 [in]  = 1 [in] 	Tables J3.4, J3.5

DESIGN CHECK

Verification	Unit	Capacity	Demand	Ctrl EQ	Ratio	References

Directly bolted

Bolts shear	[kip]	71.60	27.37	LC-2053-0	0.38    	Tables (7-1..14)

Rn = *Fnv*Ab = 0.75*54000 [lb/in2] *0.442 [in2]  = 17.901 [kip] 	Eq. J3-1

Rn = C*Rn = 4*17.901 [kip]  = 71.604 [kip] 	Tables (7-1..14)

Bolt bearing under shear load	[kip]	115.32	27.37	LC-2053-0	0.24    	Eq. J3-6

Lc-end = Max(0.0, Le - dh/2) = Max(0.0, 1.5 [in]  - 0.813 [in] /2) = 1.094 [in] 	Sec. J3.10

Lc-spa = Max(0.0, s - dh) = Max(0.0, 3 [in]  - 0.813 [in] ) = 2.188 [in] 	Sec. J3.10

Rn = *(min(k1*Lc-end, k2*d) + min(k1*Lc-spa, k2*d)*(n - 1))*tp*Fu*nc = 0.75*(min(1.2*1.094 [in] , 2.4*0.75 [in] ) + min(1.2*2.188 [in] , 2.4*0.75 [in] )*(2 - 1))*0.38 [in] *65000 [lb/in2] *2 = 115.318 [kip] 	Eq. J3-6

Block shear rupture at brace web	[kip]	119.30	27.37	LC-2053-0	0.23    	Eq. J4-5

Lh = dh + 1/16  [in]  = 0.813 [in]  + 1/16  [in]  = 0.875 [in] 	Sec. D3-2

Ant = 2 * (tp*(Lev + (n - 1)*(g - Lh) - Lh/2)) = 2 * (0.38 [in] *(1.745 [in]  + (1 - 1)*(3 [in]  - 0.875 [in] ) - 0.875 [in] /2)) = 0.994 [in2] 	Sec. J4.3

Agv = 2 * ((Lev + (n - 1)*s)*tp) = 2 * ((1.5 [in]  + (2 - 1)*3 [in] )*0.38 [in] ) = 3.42 [in2] 	Sec. J4-3

Lh = dh + 1/16  [in]  = 0.813 [in]  + 1/16  [in]  = 0.875 [in] 	Sec. D3-2

Anv = Agv - nc*(n - 0.5)*Lh*tp = 3.42 [in2]  - 2*(2 - 0.5)*0.875 [in] *0.38 [in]  = 2.422 [in2] 	Sec. J4-3

Rn = *min(0.6*Fu*Anv + Ubs*Fu*Ant, 0.6*Fy*Agv + Ubs*Fu*Ant) = 0.75*min(0.6*65000 [lb/in2] *2.422 [in2]  + 1*65000 [lb/in2] *0.994 [in2] , 0.6*50000 [lb/in2] *3.42 [in2]  + 1*65000 [lb/in2] *0.994 [in2] ) = 119.301 [kip] 	Eq. J4-5

Gusset

Bolt bearing on gusset	[kip]	113.80	27.37	LC-2053-0	0.24    	Eq. J3-6

Lc-end = Max(0.0, Le - dh/2) = Max(0.0, 1.5 [in]  - 0.813 [in] /2) = 1.094 [in] 	Sec. J3.10

Lc-spa = Max(0.0, s - dh) = Max(0.0, 3 [in]  - 0.813 [in] ) = 2.188 [in] 	Sec. J3.10

Rn = *(min(k1*Lc-end, k2*d) + min(k1*Lc-spa, k2*d)*(n - 1))*tp*Fu*nc = 0.75*(min(1.2*1.094 [in] , 2.4*0.75 [in] ) + min(1.2*2.188 [in] , 2.4*0.75 [in] )*(2 - 1))*0.375 [in] *65000 [lb/in2] *2 = 113.801 [kip] 	Eq. J3-6

Block shear on gusset	[kip]	108.77	27.37	LC-2053-0	0.25    	Eq. J4-5

Lh = dh + 1/16  [in]  = 0.813 [in]  + 1/16  [in]  = 0.875 [in] 	Sec. D3-2

Ant = tp*((n - 1)*(g - Lh)) = 0.375 [in] *((2 - 1)*(3 [in]  - 0.875 [in] )) = 0.797 [in2] 	Sec. J4.3

Agv = 2*tp*(Leh + (nc - 1)*s) = 2*0.375 [in] *(1.5 [in]  + (2 - 1)*3 [in] ) = 3.375 [in2] 	Sec. J4.3

Lh = dh + 1/16  [in]  = 0.813 [in]  + 1/16  [in]  = 0.875 [in] 	Sec. D3-2

Anv = 2*tp*(Leh + (nc - 1)*(s - Lh) - Lh/2) = 2*0.375 [in] *(1.5 [in]  + (2 - 1)*(3 [in]  - 0.875 [in] ) - 0.875 [in] /2) = 2.391 [in2] 	Sec. J4.3

Rn = *min(0.6*Fu*Anv + Ubs*Fu*Ant, 0.6*Fy*Agv + Ubs*Fu*Ant) = 0.75*min(0.6*65000 [lb/in2] *2.391 [in2]  + 1*65000 [lb/in2] *0.797 [in2] , 0.6*50000 [lb/in2] *3.375 [in2]  + 1*65000 [lb/in2] *0.797 [in2] ) = 108.773 [kip] 	Eq. J4-5

Ratio	0.38

Checks for gusset and brace

DESIGN CHECK

Member

Yielding strength due to axial load	[kip]	170.18	27.37	LC-2053-0	0.16    	Eq. J4-1

Rn = *Fy*Ag = 0.9*50000 [lb/in2] *3.782 [in2]  = 170.181 [kip] 	Eq. J4-1

Tension rupture	[kip]	96.04	27.37	LC-2053-0	0.28    	Eq. J4-2

Lh = dh + 1/16  [in]  = 0.813 [in]  + 1/16  [in]  = 0.875 [in] 	Sec. D3-2

Ahf = n*tf*Lh = 2*0.23 [in] *0.875 [in]  = 0.403 [in2] 	Sec. D3.2

An = Ag - Ahf - Ahw = 3.782 [in2]  - 0.403 [in2]  - 0 [in2]  = 3.379 [in2] 	Sec. D3.2

U = 1 - x/l = 1 - 1.251 [in] /3 [in]  = 0.583	Table D3.1

Ae = An*U = 3.379 [in2] *0.583 = 1.97 [in2] 	Eq. D3-1

Rn = *Fu*Ae = 0.75*65000 [lb/in2] *1.97 [in2]  = 96.043 [kip] 	Eq. J4-2

Gusset

Tension yielding on the Whitmore section	[kip]	109.08	27.37	LC-2053-0	0.25    	Eq. J4-1

Ag = Lw*tp = 6.464 [in] *0.375 [in]  = 2.424 [in2]

Rn = *Fy*Ag = 0.9*50000 [lb/in2] *2.424 [in2]  = 109.082 [kip] 	Eq. J4-1

Ratio	0.28

Calculation of the interface forces

Load condition :LC-2053	DG29 p. 24-33

General case		DG29 p. 24-33

K = eb*tan - ec = 1.5 [in] *0.857 - 0.207 [in]  = 1.078 [in] 		p. 13-10

K' = bar*(tan + bar/bar) = 4 [in] *(0.857 + 4 [in] /7.5 [in] ) = 5.562 [in] 		p. 13-10

D = (tan)2 + (bar/bar)2 = (0.857)2 + (4 [in] /7.5 [in] )2 = 1.019		p. 13-10

 = (K'*tan + K*(bar/bar)2)/D = (5.562 [in] *0.857 + 1.078 [in] *(4 [in] /7.5 [in] )2)/1.019 = 4.979 [in] 		p. 13-10

 = (K' - K*tan)/D = (5.562 [in]  - 1.078 [in] *0.857)/1.019 = 4.551 [in] 		p. 13-10

r = (( + ec)2 + ( + eb)2)1/2 = ((4.979 [in]  + 0.207 [in] )2 + (4.551 [in]  + 1.5 [in] )2)1/2 = 7.969 [in] 		p. 13-5

Hb = *P/r = 4.979 [in] *27.369 [kip] /7.969 [in]  = 17.099 [kip] 		p. 13-5

Hc = ec*P/r = 0.207 [in] *27.369 [kip] /7.969 [in]  = 0.713 [kip] 		p. 13-5

Vb = eb*P/r - V = 1.5 [in] *27.369 [kip] /7.969 [in]  - 0 [kip]  = 5.152 [kip] 		p. 13-5

Vc = *P/r + V = 4.551 [in] *27.369 [kip] /7.969 [in]  + 0 [kip]  = 15.629 [kip] 		p. 13-5

Mb = abs(Vb*( - bar)) + abs(V*) = abs(5.152 [kip] *(4.979 [in]  - 4 [in] )) + abs(0 [kip] *4.979 [in] ) = 0.42 [kip*ft] 		p. 13-10

Mc = abs(Hc*( - bar)) = abs(0.713 [kip] *(4.551 [in]  - 7.5 [in] )) = 0.175 [kip*ft] 		p. 13-10

Left gusset interface - base plate

Directly welded

DEMANDS

Description	Ru	Pu	Mu	Pu	Mu22	Mu33	Load type

kip	kip	kip*ft	kip	kip*ft	kip*ft

LC-2053	17.10	5.15	0.42	0.00	0.00	0.00	Design

DESIGN CHECK

Verification	Unit	Capacity	Demand	Ctrl EQ	Ratio	References

Gusset

Beam yielding (normal stress)	[kip]	135.00	7.67	LC-2053-0	0.06    	Eq. B-1, Appendix B, DG29, Eq. J4-1

Neq = Vub + ((4*Mub)/Lp) = 5.152 [kip]  + ((4*0.42 [kip*ft] )/8 [in] ) = 7.673 [kip] 	Eq. B-1, Appendix B, DG29

Ag = Lp*tp = 8 [in] *0.375 [in]  = 3 [in2] 	Sec. D3-1

Rn = *Fy*Ag = 0.9*50000 [lb/in2] *3 [in2]  = 135 [kip] 	Eq. J4-1

Neq = Vub + ((4*Mub)/Lp) = 5.152 [kip]  + ((4*0.42 [kip*ft] )/8 [in] ) = 7.673 [kip] 	Eq. B-1, Appendix B, DG29

Shear yielding	[kip]	90.00	17.10	LC-2053-0	0.19    	Eq. J4-3

Ag = Lp*tp = 8 [in] *0.375 [in]  = 3 [in2] 	Sec. D3-1

Rn = *0.60*Fy*Ag = 1*0.60*50000 [lb/in2] *3 [in2]  = 90 [kip] 	Eq. J4-3

Gusset edge tension stress	[ksi]	45.00	1.72	LC-2053-0	0.04    	J4-1

Fn = *Fy = 0.9*50000 [lb/in2]  = 45000 [lb/in2] 	J4-1

fua = Hc/(tp*l) = 5.152 [kip] /(0.375 [in] *8 [in] ) = 1717.18 [lb/in2] 	[9]

Gusset edge shear stress	[ksi]	30.00	5.70	LC-2053-0	0.19    	J4-1

Fn = *0.6*Fy = 1*0.6*50000 [lb/in2]  = 30000 [lb/in2] 	J4-1

fuv = Vc/(tp*l) = 17.099 [kip] /(0.375 [in] *8 [in] ) = 5699.61 [lb/in2] 	[9]

Weld capacity	[kip]	128.92	22.78	LC-2053-0	0.18    	Tables 8-4 .. 8-11

Rn = 2 * (*C*C1*D*L) = 2 * (0.75*2.149 [kip/in] *1*5*8 [in] ) = 128.916 [kip] 	Tables 8-4 .. 8-11

fua = Vc/l = 5.152 [kip] /8 [in]  = 0.644 [kip/in] 	[9]

fuv = Hc/l = 17.099 [kip] /8 [in]  = 2.137 [kip/in] 	[9]

fub = Mc/(l2/6) = 0.42 [kip*ft] /(8 [in] 2/6) = 0.473 [kip/in] 	[9]

fuPeak = ((fua + fub)2 + fuv2)1/2 = ((0.644 [kip/in]  + 0.473 [kip/in] )2 + 2.137 [kip/in] 2)1/2 = 2.411 [kip/in] 	[9]

fuAve = 0.5*(((fua - fub)2 + fuv2)1/2 + ((fua + fub)2 + fuv2)1/2) = 0.5*(((0.644 [kip/in]  - 0.473 [kip/in] )2 + 2.137 [kip/in] 2)1/2 + ((0.644 [kip/in]  + 0.473 [kip/in] )2 + 2.137 [kip/in] 2)1/2) = 2.278 [kip/in] 	[9]

fuWeld = l*max(fuPeak, 1.25*fuAve) = 8 [in] *max(2.411 [kip/in] , 1.25*2.278 [kip/in] ) = 22.778 [kip] 	[9]

Ratio	0.19

Upper left gusset interface - column

Directly welded

DEMANDS

Description	Ru	Pu	Mu	Pu	Mu22	Mu33	Load type

kip	kip	kip*ft	kip	kip*ft	kip*ft

LC-2053	15.63	0.71	0.18	32.82	0.00	0.00	Design

DESIGN CHECK

Gusset

Beam yielding (normal stress)	[kip]	253.13	1.27	LC-2053-0	0.01    	Eq. B-1, Appendix B, DG29, Eq. J4-1

Neq = Vub + ((4*Mub)/Lp) = 0.713 [kip]  + ((4*0.175 [kip*ft] )/15 [in] ) = 1.273 [kip] 	Eq. B-1, Appendix B, DG29

Ag = Lp*tp = 15 [in] *0.375 [in]  = 5.625 [in2] 	Sec. D3-1

Rn = *Fy*Ag = 0.9*50000 [lb/in2] *5.625 [in2]  = 253.125 [kip] 	Eq. J4-1

Neq = Vub + ((4*Mub)/Lp) = 0.713 [kip]  + ((4*0.175 [kip*ft] )/15 [in] ) = 1.273 [kip] 	Eq. B-1, Appendix B, DG29

Shear yielding	[kip]	168.75	15.63	LC-2053-0	0.09    	Eq. J4-3

Ag = Lp*tp = 15 [in] *0.375 [in]  = 5.625 [in2] 	Sec. D3-1

Rn = *0.60*Fy*Ag = 1*0.60*50000 [lb/in2] *5.625 [in2]  = 168.75 [kip] 	Eq. J4-3

Gusset edge tension stress	[ksi]	45.00	0.13	LC-2053-0	0.00    	J4-1

Fn = *Fy = 0.9*50000 [lb/in2]  = 45000 [lb/in2] 	J4-1

fua = Hc/(tp*l) = 0.713 [kip] /(0.375 [in] *15 [in] ) = 126.69 [lb/in2] 	[9]

Gusset edge shear stress	[ksi]	30.00	2.78	LC-2053-0	0.09    	J4-1

Fn = *0.6*Fy = 1*0.6*50000 [lb/in2]  = 30000 [lb/in2] 	J4-1

fuv = Vc/(tp*l) = 15.629 [kip] /(0.375 [in] *15 [in] ) = 2778.4 [lb/in2] 	[9]

Weld capacity	[kip]	212.07	19.58	LC-2053-0	0.09    	Tables 8-4 .. 8-11

Rn = 2 * (*C*C1*D*L) = 2 * (0.75*1.885 [kip/in] *1*5*15 [in] ) = 212.065 [kip] 	Tables 8-4 .. 8-11

fua = Vc/l = 0.713 [kip] /15 [in]  = 0.0475 [kip/in] 	[9]

fuv = Hc/l = 15.629 [kip] /15 [in]  = 1.042 [kip/in] 	[9]

fub = Mc/(l2/6) = 0.175 [kip*ft] /(15 [in] 2/6) = 0.056 [kip/in] 	[9]

fuPeak = ((fua + fub)2 + fuv2)1/2 = ((0.0475 [kip/in]  + 0.056 [kip/in] )2 + 1.042 [kip/in] 2)1/2 = 1.047 [kip/in] 	[9]

fuAve = 0.5*(((fua - fub)2 + fuv2)1/2 + ((fua + fub)2 + fuv2)1/2) = 0.5*(((0.0475 [kip/in]  - 0.056 [kip/in] )2 + 1.042 [kip/in] 2)1/2 + ((0.0475 [kip/in]  + 0.056 [kip/in] )2 + 1.042 [kip/in] 2)1/2) = 1.044 [kip/in] 	[9]

fuWeld = l*max(fuPeak, 1.25*fuAve) = 15 [in] *max(1.047 [kip/in] , 1.25*1.044 [kip/in] ) = 19.584 [kip] 	[9]

Column

Web plate strength due out-of-plane transverse load	[kip]	36.51	0.71	LC-2053-0	0.02    	p.9-16

Rn = *tw2*(Fy/4.0)*(4.0*(2.0*T*a*b*(a + b))0.5 + L*(a + b))/(a*b) = 1*0.415 [in] 2*(50000 [lb/in2] /4.0)*(4.0*(2.0*11.38 [in] *5.503 [in] *5.503 [in] *(5.503 [in]  + 5.503 [in] ))0.5 + 15 [in] *(5.503 [in]  + 5.503 [in] ))/(5.503 [in] *5.503 [in] ) = 36.505 [kip] 	p.9-16

Ratio	0.09

interface between Column - base plate

DEMANDS

Description	Pu	Mu22	Mu33	Vu2	Vu3	Load type

kip	kip*ft	kip*ft	kip	kip

LC-2053	53.60	7.72	-6.57	0.67	16.86	Design

Design for minor axis

Base plate (AISC 360-16 LRFD)

GEOMETRIC CONSIDERATIONS

Dimensions	Unit	Value	Min.	Max.	Sta.	References

Base plate

Distance from anchor to edge	[in]	4.75	0.25	-

Weld size	[1/16in]	6	3.00	-	    	table J2.4

Left gusset

Weld size	[1/16in]	5	3.00	-	    	table J2.4

DESIGN CHECK

Verification	Unit	Capacity	Demand	Ctrl EQ	Ratio	References

Pedestal

Axial bearing	[ksi]	4.42	0.00	LC-2053-0	0.00    	DG1 3.1.1;

A2 = ((B/N)*Ncs)*Ncs = ((18 [in] /18 [in] )*36 [in] )*36 [in]  = 1296 [in2] 	DG1 Sec 3.1.1

A1 = B*N = 18 [in] *18 [in]  = 324 [in2] 	DG1 Sec 3.1.1

fp, max = *min(0.85*f'c*(A2/A1)1/2, 1.7*f'c) = 0.65*min(0.85*4000 [lb/in2] *(4)1/2, 1.7*4000 [lb/in2] ) = 4420 [lb/in2] 	DG1 3.1.1

Base plate

Flexural yielding (bearing interface)	[kip*ft/ft]	25.31	0.00	LC-2053-0	0.00    	DG1 Eq. 3.3.13

Mn = *Fy*tp2/4 = 0.9*50000 [lb/in2] *1.5 [in] 2/4 = 25.313 [kip*ft/ft] 	DG1 Eq. 3.3.13

Flexural yielding (tension interface)	[kip*ft/ft]	25.31	10.01	LC-2053-0	0.40    	DG1 Eq. 3.3.13

Mn = *Fy*tp2/4 = 0.9*50000 [lb/in2] *1.5 [in] 2/4 = 25.313 [kip*ft/ft] 	DG1 Eq. 3.3.13

MpT = Mstrip/Beff = 1.301 [kip*ft] /1.56 [in]  = 10.007 [kip*ft/ft]

Column

Weld capacity	[kip/in]	12.53	12.83	LC-2053-0	1.02    	p. 8-9, Sec. J2.5, Sec. J2.4, DG1 p. 35

LoadAngleFactor = 1 + 0.5*(sin())1.5 = 1 + 0.5*(sin(1.571))1.5 = 1.5	p. 8-9

Fw = 0.6*FEXX*LoadAngleFactor = 0.6*70000 [lb/in2] *1.5 = 63000 [lb/in2] 	Sec. J2.5

Aw = (2)1/2/2*D/16  [in] *L = (2)1/2/2*6/16  [in] *1 [in]  = 0.265 [in2] 	Sec. J2.4

Rw = *Fw*Aw/L = 0.75*63000 [lb/in2] *0.265 [in2] /1 [in]  = 12.529 [kip/in]

beff = 2*L = 2*0.78 [in]  = 1.56 [in] 	DG1 p. 35

Maximum weld load = T/beff = 20.014 [kip] /1.56 [in]  = 12.83 [kip/in]

WARNINGS

High value moments detected in the axis perpendicular to the design axis

Ratio	1.02

Minor axis

Anchors

GEOMETRIC CONSIDERATIONS

Dimensions	Unit	Value	Min.	Max.	Sta.	References

Anchors

Anchor spacing	[in]	7.00	6.00	-	    	Sec. D.8.1

smin = 4*da = 4*1.5 [in]  = 6 [in] 	Sec. D.8.1

Concrete cover	[in]	13.75	2.00	-	    	Sec. 7.7.1

IsConcreteCastAgainstEarthFalse

Cover = 2  [in] 	Sec. 7.7.1

Effective length	[in]	20.98	-	47.03

Verification	Unit	Capacity	Demand	Ctrl EQ	Ratio	References

DESIGN CHECK

Anchor tension	[kip]	61.13	20.01	LC-2053-0	0.33  	Eq. D-2

Ase = π/4.0*(da - 0.9743  [in] /nt)2 = π/4.0*(1.5 [in]  - 0.9743  [in] /6)2 = 1.405 [in2] 	Sec. D.5.1.1, D.6.1.2

futa = min(futa, 1.9*fya, 125  [ksi] ) = min(58000 [lb/in2] , 1.9*36000 [lb/in2] , 125  [ksi] ) = 58000 [lb/in2] 	Sec. D.5.1.2

Nsa = *Ase,N*futa = 0.75*1.405 [in2] *58000 [lb/in2]  = 61.128 [kip] 	Eq. D-2

Pullout of anchor in tension	[kip]	58.63	20.01	LC-2053-0	0.34  	Sec. D.3.3.4.4

Abrg = 0.866025*F2 - Ag = 0.866025*2.25 [in] 2 - 1.767 [in2]  = 2.617 [in2]

IsHeadedBoltTrue

Np = 8*Abrg*fc = 8*2.617 [in2] *4000 [lb/in2]  = 83.752 [kip] 	Eq. D-14

CrackedConcreteTrue

c,P = 1	Sec. D.5.3.6

Npn = c,P*Np = 1*83.752 [kip]  = 83.752 [kip] 	Eq. D-13

HighSeismicDesignCategoryFalse

Npn = *Npn = 0.7*83.752 [kip]  = 58.626 [kip] 	Sec. D.3.3.4.4

Anchors reinforcement in tension	[kip]	237.60	53.60	LC-2053-0	0.23  	Sec. D.5.2.9, D.6.2.9

Nsar = 0.75*n*As*Fy = 0.75*12*0.44 [in2] *60000 [lb/in2]  = 237.6 [kip] 	Sec. D.5.2.9, D.6.2.9

Anchor shear	[kip]	25.43	4.22	LC-2053-0	0.17  	Eq. D-29, Sec. D.6.1.3

Ase = π/4.0*(da - 0.9743  [in] /nt)2 = π/4.0*(1.5 [in]  - 0.9743  [in] /6)2 = 1.405 [in2] 	Sec. D.5.1.1, D.6.1.2

futa = min(futa, 1.9*fya, 125  [ksi] ) = min(58000 [lb/in2] , 1.9*36000 [lb/in2] , 125  [ksi] ) = 58000 [lb/in2] 	Sec. D.5.1.2

HasGroutPadTrue

Vsa = 0.8**0.6*n*Ase,V*futa = 0.8*0.65*0.6*1*1.405 [in2] *58000 [lb/in2]  = 25.429 [kip] 	Eq. D-29, Sec. D.6.1.3

Pryout of anchor in shear	[kip]	75.78	4.22	LC-2053-0	0.06  	Eq. D-3, Table D.4.1.1, Sec. D.4.3

hef<2.5  [in] 20 [in] <2.5  [in] False

kcp = 2	Sec. D.6.3.1

ca1Left<1.5*hef14.5 [in] <1.5*20 [in] True

ca1Left = ca1Left = 14.5 [in] 	Sec. D.5.2.1

ca1Right<1.5*hef21.5 [in] <1.5*20 [in] True

ca1Right = ca1Right = 21.5 [in] 	Sec. D.5.2.1

ca2Top<1.5*hef14.5 [in] <1.5*20 [in] True

ca2Top = ca2Top = 14.5 [in] 	Sec. D.5.2.1

ca2Bot<1.5*hef21.5 [in] <1.5*20 [in] True

ca2Bot = ca2Bot = 21.5 [in] 	Sec. D.5.2.1

IsCloseToThreeEdgesTrue

hef = camax/1.5 = 21.5 [in] /1.5 = 14.333 [in] 	Sec. D.5.2.3

ca1Left<1.5*hef14.5 [in] <1.5*14.333 [in] True

ca1Left = ca1Left = 14.5 [in] 	Sec. D.5.2.1

ca1Right<1.5*hef21.5 [in] <1.5*14.333 [in] False

ca1Right = 1.5*hef = 1.5*14.333 [in]  = 21.5 [in] 	Sec. D.5.2.1

ca2Top<1.5*hef14.5 [in] <1.5*14.333 [in] True

ca2Top = ca2Top = 14.5 [in] 	Sec. D.5.2.1

ca2Bot<1.5*hef21.5 [in] <1.5*14.333 [in] False

ca2Bot = 1.5*hef = 1.5*14.333 [in]  = 21.5 [in] 	Sec. D.5.2.1

ANc = (ca1Left + ca1Right)*(ca2Top + ca2Bot) = (14.5 [in]  + 21.5 [in] )*(14.5 [in]  + 21.5 [in] ) = 1296 [in2] 	Sec. RD.5.2.1

ANco = 9*hef2 = 9*14.333 [in] 2 = 1849 [in2] 	Eq. D-5

ca,min<1.5*hef14.5 [in] <1.5*14.333 [in] True

ed,N = 0.7 + 0.3*ca,min/(1.5*hef) = 0.7 + 0.3*14.5 [in] /(1.5*14.333 [in] ) = 0.902	Eq. D-10

CrackedConcreteTrue

c,N = 1	Sec. D.5.2.6

IsCastInPlaceAnchorTrue

cp,N = 1	Sec. D.5.2.7

IsCastInPlaceAnchorTrue

kc = 24	Sec. D.5.2.2

(IsCastInPlaceAnchor) and (IsHeadedBolt) and (hef>=11  [in] ) and (hef<=25  [in] )(True) and (True) and (14.333 [in] >=11  [in] ) and (14.333 [in] <=25  [in] )True

Nb = 16*a*(fc/(1  [psi] ))1/2*(hef/(1  [in] ))(5/3)  [lb]  = 16*1*(4000 [lb/in2] /(1  [psi] ))1/2*(14.333 [in] /(1  [in] ))(5/3)  [lb]  = 85.585 [kip] 	Eq. D-7

Ncb = (ANc/ANco)*ed,N*c,N*cp,N*Nb = (1296 [in2] /1849 [in2] )*0.902*1*1*85.585 [kip]  = 54.129 [kip] 	Eq. D-3

Vcp = kcp*Ncb = 2*54.129 [kip]  = 108.257 [kip] 	Eq. D-40

Vcp = *Vcp = 0.7*108.257 [kip]  = 75.78 [kip] 	Table D.4.1.1, Sec. D.4.3

Pryout of group of anchors in shear	[kip]	87.94	16.86	LC-2053-0	0.19  	Eq. D-4, Table D.4.1.1, Sec. D.4.3

hef<2.5  [in] 20 [in] <2.5  [in] False

kcp = 2	Sec. D.6.3.1

ANco = 9*hef2 = 9*9.667 [in] 2 = 841 [in2] 	Eq. D-5

ANc = min(ANc, n*ANco) = min(1296 [in2] , 4*841 [in2] ) = 1296 [in2] 	Sec. D.5.2.1

ec,Ny = min(1/(1 + 2*e'N/(3*hef)), 1) = min(1/(1 + 2*1.728 [in] /(3*9.667 [in] )), 1) = 0.894	Eq. D-8

ec,Nx = min(1/(1 + 2*e'N/(3*hef)), 1) = min(1/(1 + 2*0 [in] /(3*9.667 [in] )), 1) = 1	Eq. D-8

ec,N = ec,Nx*ec,Ny = 1*0.894 = 0.894	Eq. D-8

ca,min<1.5*hef14.5 [in] <1.5*9.667 [in] False

ed,N = 1	Eq. D-9

CrackedConcreteTrue

c,N = 1	Sec. D.5.2.6

IsCastInPlaceAnchorTrue

cp,N = 1	Sec. D.5.2.7

IsCastInPlaceAnchorTrue

kc = 24	Sec. D.5.2.2

(IsCastInPlaceAnchor) and (IsHeadedBolt) and (hef>=11  [in] ) and (hef<=25  [in] )(True) and (True) and (9.667 [in] >=11  [in] ) and (9.667 [in] <=25  [in] )False

Nb = kc*a*(fc/(1  [psi] ))1/2*(hef/(1  [in] ))1.5  [lb]  = 24*1*(4000 [lb/in2] /(1  [psi] ))1/2*(9.667 [in] /(1  [in] ))1.5  [lb]  = 45.62 [kip] 	Eq. D-6

Ncbg = (ANc/ANco)*ec,N*ed,N*c,N*cp,N*Nb = (1296 [in2] /841 [in2] )*0.894*1*1*1*45.62 [kip]  = 62.818 [kip] 	Eq. D-4

Vcpg = kcp*Ncbg = 2*62.818 [kip]  = 125.635 [kip] 	Eq. D-41

Vcpg = *Vcpg = 0.7*125.635 [kip]  = 87.945 [kip] 	Table D.4.1.1, Sec. D.4.3

Anchors reinforcement in shear	[kip]	36.00	16.86	LC-2053-0	0.47  	Sec. D.5.2.9, D.6.2.9

Nsar = 0.75*n*As*Fy = 0.75*4*0.2 [in2] *60000 [lb/in2]  = 36 [kip] 	Sec. D.5.2.9, D.6.2.9

Interaction of tensile and shear forces	[kip]	1.20	0.00	LC-2053-0	0.00  	Eq. D-2, Sec. D.3.3.4.4, Eq. D-29, Sec. D.6.1.3, Eq. D-3, Table D.4.1.1, Sec. D.4.3, Eq. D-4, Sec. D.7

Ase = π/4.0*(da - 0.9743  [in] /nt)2 = π/4.0*(1.5 [in]  - 0.9743  [in] /6)2 = 1.405 [in2] 	Sec. D.5.1.1, D.6.1.2

futa = min(futa, 1.9*fya, 125  [ksi] ) = min(58000 [lb/in2] , 1.9*36000 [lb/in2] , 125  [ksi] ) = 58000 [lb/in2] 	Sec. D.5.1.2

Nsa = *Ase,N*futa = 0.75*1.405 [in2] *58000 [lb/in2]  = 61.128 [kip] 	Eq. D-2

Abrg = 0.866025*F2 - Ag = 0.866025*2.25 [in] 2 - 1.767 [in2]  = 2.617 [in2]

IsHeadedBoltTrue

Np = 8*Abrg*fc = 8*2.617 [in2] *4000 [lb/in2]  = 83.752 [kip] 	Eq. D-14

CrackedConcreteTrue

c,P = 1	Sec. D.5.3.6

Npn = c,P*Np = 1*83.752 [kip]  = 83.752 [kip] 	Eq. D-13

HighSeismicDesignCategoryFalse

Npn = *Npn = 0.7*83.752 [kip]  = 58.626 [kip] 	Sec. D.3.3.4.4

SideFaceBlowoutApply = hef>2.5*ca1 = 20 [in] >2.5*14.5 [in]  = False	Sec. D.5.4.1

Ase = π/4.0*(da - 0.9743  [in] /nt)2 = π/4.0*(1.5 [in]  - 0.9743  [in] /6)2 = 1.405 [in2] 	Sec. D.5.1.1, D.6.1.2

futa = min(futa, 1.9*fya, 125  [ksi] ) = min(58000 [lb/in2] , 1.9*36000 [lb/in2] , 125  [ksi] ) = 58000 [lb/in2] 	Sec. D.5.1.2

HasGroutPadTrue

Vsa = 0.8**0.6*n*Ase,V*futa = 0.8*0.65*0.6*1*1.405 [in2] *58000 [lb/in2]  = 25.429 [kip] 	Eq. D-29, Sec. D.6.1.3

hef<2.5  [in] 20 [in] <2.5  [in] False

kcp = 2	Sec. D.6.3.1

ca1Left<1.5*hef14.5 [in] <1.5*20 [in] True

ca1Left = ca1Left = 14.5 [in] 	Sec. D.5.2.1

ca1Right<1.5*hef21.5 [in] <1.5*20 [in] True

ca1Right = ca1Right = 21.5 [in] 	Sec. D.5.2.1

ca2Top<1.5*hef14.5 [in] <1.5*20 [in] True

ca2Top = ca2Top = 14.5 [in] 	Sec. D.5.2.1

ca2Bot<1.5*hef21.5 [in] <1.5*20 [in] True

ca2Bot = ca2Bot = 21.5 [in] 	Sec. D.5.2.1

IsCloseToThreeEdgesTrue

hef = camax/1.5 = 21.5 [in] /1.5 = 14.333 [in] 	Sec. D.5.2.3

ca1Left<1.5*hef14.5 [in] <1.5*14.333 [in] True

ca1Left = ca1Left = 14.5 [in] 	Sec. D.5.2.1

ca1Right<1.5*hef21.5 [in] <1.5*14.333 [in] False

ca1Right = 1.5*hef = 1.5*14.333 [in]  = 21.5 [in] 	Sec. D.5.2.1

ca2Top<1.5*hef14.5 [in] <1.5*14.333 [in] True

ca2Top = ca2Top = 14.5 [in] 	Sec. D.5.2.1

ca2Bot<1.5*hef21.5 [in] <1.5*14.333 [in] False

ca2Bot = 1.5*hef = 1.5*14.333 [in]  = 21.5 [in] 	Sec. D.5.2.1

ANc = (ca1Left + ca1Right)*(ca2Top + ca2Bot) = (14.5 [in]  + 21.5 [in] )*(14.5 [in]  + 21.5 [in] ) = 1296 [in2] 	Sec. RD.5.2.1

ANco = 9*hef2 = 9*14.333 [in] 2 = 1849 [in2] 	Eq. D-5

ca,min<1.5*hef14.5 [in] <1.5*14.333 [in] True

ed,N = 0.7 + 0.3*ca,min/(1.5*hef) = 0.7 + 0.3*14.5 [in] /(1.5*14.333 [in] ) = 0.902	Eq. D-10

CrackedConcreteTrue

c,N = 1	Sec. D.5.2.6

IsCastInPlaceAnchorTrue

cp,N = 1	Sec. D.5.2.7

IsCastInPlaceAnchorTrue

kc = 24	Sec. D.5.2.2

(IsCastInPlaceAnchor) and (IsHeadedBolt) and (hef>=11  [in] ) and (hef<=25  [in] )(True) and (True) and (14.333 [in] >=11  [in] ) and (14.333 [in] <=25  [in] )True

Nb = 16*a*(fc/(1  [psi] ))1/2*(hef/(1  [in] ))(5/3)  [lb]  = 16*1*(4000 [lb/in2] /(1  [psi] ))1/2*(14.333 [in] /(1  [in] ))(5/3)  [lb]  = 85.585 [kip] 	Eq. D-7

Ncb = (ANc/ANco)*ed,N*c,N*cp,N*Nb = (1296 [in2] /1849 [in2] )*0.902*1*1*85.585 [kip]  = 54.129 [kip] 	Eq. D-3

Vcp = kcp*Ncb = 2*54.129 [kip]  = 108.257 [kip] 	Eq. D-40

Vcp = *Vcp = 0.7*108.257 [kip]  = 75.78 [kip] 	Table D.4.1.1, Sec. D.4.3

hef<2.5  [in] 20 [in] <2.5  [in] False

kcp = 2	Sec. D.6.3.1

ANco = 9*hef2 = 9*9.667 [in] 2 = 841 [in2] 	Eq. D-5

ANc = min(ANc, n*ANco) = min(1296 [in2] , 4*841 [in2] ) = 1296 [in2] 	Sec. D.5.2.1

ec,Ny = min(1/(1 + 2*e'N/(3*hef)), 1) = min(1/(1 + 2*1.728 [in] /(3*9.667 [in] )), 1) = 0.894	Eq. D-8

ec,Nx = min(1/(1 + 2*e'N/(3*hef)), 1) = min(1/(1 + 2*0 [in] /(3*9.667 [in] )), 1) = 1	Eq. D-8

ec,N = ec,Nx*ec,Ny = 1*0.894 = 0.894	Eq. D-8

ca,min<1.5*hef14.5 [in] <1.5*9.667 [in] False

ed,N = 1	Eq. D-9

CrackedConcreteTrue

c,N = 1	Sec. D.5.2.6

IsCastInPlaceAnchorTrue

cp,N = 1	Sec. D.5.2.7

IsCastInPlaceAnchorTrue

kc = 24	Sec. D.5.2.2

(IsCastInPlaceAnchor) and (IsHeadedBolt) and (hef>=11  [in] ) and (hef<=25  [in] )(True) and (True) and (9.667 [in] >=11  [in] ) and (9.667 [in] <=25  [in] )False

Nb = kc*a*(fc/(1  [psi] ))1/2*(hef/(1  [in] ))1.5  [lb]  = 24*1*(4000 [lb/in2] /(1  [psi] ))1/2*(9.667 [in] /(1  [in] ))1.5  [lb]  = 45.62 [kip] 	Eq. D-6

Ncbg = (ANc/ANco)*ec,N*ed,N*c,N*cp,N*Nb = (1296 [in2] /841 [in2] )*0.894*1*1*1*45.62 [kip]  = 62.818 [kip] 	Eq. D-4

Vcpg = kcp*Ncbg = 2*62.818 [kip]  = 125.635 [kip] 	Eq. D-41

Vcpg = *Vcpg = 0.7*125.635 [kip]  = 87.945 [kip] 	Table D.4.1.1, Sec. D.4.3

(Nua>0.2*Nn) and (Vua>0.2*Vn)(20.014 [kip] >0.2*58.626 [kip] ) and (16.861 [kip] >0.2*87.945 [kip] )False

TensionShearInteraction = 0	Sec. D.7

Ratio	0.47

Ratio	1.02

Minor axis

Maximum compression and tension (LC-2053)

lb/in2

Maximum bearing pressure	0.00

Minimum bearing pressure	0.00

Maximum anchor tension	20.01

Minimum anchor tension	6.79

Neutral axis angle	0.00

Bearing length	1.91

Anchors tensions

Anchor	Transverse	Longitudinal	Shear	Tension

in	in	kip	kip

1	-3.50	-3.50	4.22	20.01

2	-3.50	3.50	4.22	20.01

3	3.50	3.50	4.22	6.79

4	3.50	-3.50	4.22	6.79

Global critical strength ratio	1.02

NOTATION

a  :	Plate depth

A1  :	Base plate area

A2  :	Maximum area of portion of the concrete supporting surface that is geometrically similar to and concentric with the load area

Ab  :	Nominal bolt area

Ae  :	Effective net area

Ag  :	Gross area

Agv  :	Gross area subject to shear

Ahf  :	Flange holes area

Ahw  :	Holes area on web

An  :	Net area

Ant  :	Net area subject to tension

Anv  :	Net area subjected to shear

Aw  :	Effective area of the weld

  :	Distance from the face of the column flange or web to the centroid of the gusset to beam connection

bar  :	Centroid of the gusset to beam connection

A2/A1  :	Ratio between the concrete support area and the base plate area

b  :	Plate, connector or member width

B  :	Base plate design width

beff  :	Effective width of the compression block

Beff  :	Controlling effective width

  :	Distance from the face of the beam flange to the centroid of the gusset to column connection

bar  :	Centroid of the gusset to column connection

C  :	Bolt group coefficient

C1  :	Electrode strength coefficient

C2  :	Edge distance increment

C  :	Weld group coefficient

d  :	Nominal bolt diameter

dh  :	Nominal hole dimension

D  :	Uniform force method factor

V  :	Arbitrary shear

D  :	Number of sixteenths of an inch in the weld size

eb  :	One half the depth of the beam

ec  :	One half the depth of the column

f'c  :	Specified compressive strength of concrete

FEXX  :	Electrode classification number

Fnv  :	Nominal shear stress

fp, max  :	Maximum uniformly bearing stress under base plate

Fu  :	Specified minimum tensile strength

fua  :	Axial stress on welds along gusset-beam or gusset-column interface

fua  :	Axial stress on welds along gusset-beam or gusset-column interface

fuAve  :	Average weld stress on welds along gusset-beam or gusset-column interface

fub  :	Bending stress on welds along gusset-beam or gusset-column interface

fuPeak  :	Peak weld stress on welds along gusset-beam or gusset-column interface

fuv  :	Shear stress on welds along gusset-beam or gusset-column interface

fuv  :	Shear stress on welds along gusset-beam or gusset-column interface

fuWeld  :	Design weld force on welds along gusset-beam or gusset-column interface

Fw  :	Nominal strength of the weld metal per unit area

Fy  :	Specified minimum yield stress

g  :	Transversal gage between bolts

Hb  :	Required shear force on the beam to gusset connection

HBeam  :	Beam horizontal force

HBeamToColumn  :	Beam to column interrface total horizontal force

HBot  :	Bottom horizontal component of the gusset forces

Hc  :	Required axial force on the column to gusset connection

HTop  :	Top horizontal component of the gusset forces

IsCorrosionConsidered  :	Is corrosion considered

k1  :	Bearing factor

k2  :	Bearing factor

K  :	Uniform force method factor

K'  :	Uniform force method factor

l  :	Length

L  :	Length

L  :	Distance from the anchor rod to the column

Lc-end  :	Clear distance

Le  :	Edge distance

Leh  :	Horizontal edge distance

Lemin  :	Minimum edge distance

Lev  :	Vertical edge distance

Lh  :	Hole dimension for tension and shear net area

Lp  :	Plate length

L  :	Length of weld

Lw  :	Width of Whitmore section

LoadAngleFactor  :	Load angle factor

Mb  :	Required moment on the beam to gusset connection

Mc  :	Required moment on the column to gusset connection

MpT  :	Plate bending moment per unit width at tension unstiffened strip interface

Mstrip  :	Maximum bending moment at the strip

Mub  :	Moment applied to the interface

Maximum weld load  :	Maximum weld load

edmin  :	Minimum edge distance

n  :	Bolts rows number

N  :	Base plate design length

nc  :	Number of bolt columns

Ncs  :	Length of the concrete supporting surface or pier parallel to moment design direction

Neq  :	Equivalent normal force

P  :	Required axial force

  :	Design factors

Fn  :	Design or allowable tension/shear yielding stress

Mn  :	Design or allowable strength per unit length

Rn  :	Design or allowable strength

Rw  :	Fillet weld capacity per unit length

r  :	Uniform force method parameter

smax  :	Maximum spacing

smin  :	Minimum spacing

s  :	Longitudinal bolt spacing

Lc-spa  :	Distance between adjacent holes edges

tp  :	Thickness of the connected material

T  :	Clear distance between web fillets

T  :	Anchor rod tensile strength required

tf  :	Thickness of the loaded flange

tp  :	Plate thickness

tw  :	Web thickness

tan  :	Tangent of the brace with the vertical angle

  :	Load angle

U  :	Shear lag factor

Ubs  :	Stress index

Vb  :	Required axial force on the beam to gusset connection

VBeam  :	Beam vertical force

VBeamToColumn  :	Beam to column interrface total vertical force

VBot  :	Bottom vertical component of the gusset forces

Vc  :	Required shear force on the column to gusset connection

VTop  :	Top vertical component of the gusset forces

Vub  :	Shear applied to the interface

wmin  :	Minimum weld size required

x  :	Connection eccentricity

Abrg  :	Net bearing area of the head of stud or anchor bolt

Ag  :	Gross area of anchor

ANc  :	Projected concrete failure area of a single anchor or group of anchors, for calculation of strength in tension

ANco  :	Projected concrete failure area of a single anchor, for calculation of strength in tension if not limited by edge distance or spacing

As  :	Effective cross-sectional area of anchor reinforcement

Ase  :	Effective cross-sectional area of anchor

Ase,N  :	Effective cross-sectional area of anchor in tension

Ase,V  :	Effective cross-sectional area of anchor in shear

ca1  :	Distance from the anchor center to the concrete edge

ca1Left  :	Distance from the anchor center to the left edge of the concrete base

ca1Right  :	Distance from the anchor center to the right edge of the concrete base

ca2Bot  :	Distance from the anchor center to the bottom edge of the concrete base

ca2Top  :	Distance from the anchor center to the top edge of the concrete base

camax  :	Maximum distance from center of an anchor shaft to the edge of concrete

ca,min  :	Minimum distance from center of an anchor shaft to the edge of concrete

Cover  :	Concrete cover

CrackedConcrete  :	Cracked concrete at service loads

da  :	Outside diameter of anchor or shaft diameter of headed stud, headed bolt, or hooked bolt

e'N  :	Distance between resultant tension load on a group of anchors loaded in tension and the centroid of the group of anchors loaded in tension

F  :	Distance between head flat sides

fc  :	Specified compressive strength of concrete

futa  :	Specified tensile strength of anchor steel

Fy  :	Specified minimum yield stress

fya  :	Specified yield strength of anchor steel

hef  :	Effective embedment depth of anchor

HasGroutPad  :	Has grout pad

HighSeismicDesignCategory  :	High seismic design category (i.e. C, D, E or F)

IsCastInPlaceAnchor  :	Is cast in place anchor

IsCloseToThreeEdges  :	Anchor is close to three or more edges

IsConcreteCastAgainstEarth  :	Is concrete cast against and permanently exposed to earth

IsHeadedBolt  :	Is anchor headed stud

kc  :	Coefficient for concrete pry out basic strength

kcp  :	Coefficient for pry out strength

a  :	Lightweight concrete modification factor

n  :	Number of anchors in the group

Nb  :	Basic concrete breakout strength in tension of a single anchor in cracked concrete

Ncb  :	Nominal concrete breakout strength in tension of a single anchor

Ncbg  :	Nominal concrete breakout strength in tension of a group of anchors

Np  :	Pullout strength in tension of a single anchor in cracked concrete

Npn  :	Nominal pullout strength of a single anchor in tension

n  :	Number of anchor reinforcement bars

nt  :	Number of threads per inch

Nua  :	Factored tensile force applied to anchor or group of anchors

  :	Strength reduction factor

Nn  :	Tension strength

Npn  :	Pullout strength in tension of a single anchor

Nsa  :	Strength of a single anchor or group of anchors in tension

Nsar  :	Strength of a single anchor reinforcement or group of anchors reinforcements in tension

Vcp  :	Concrete pryout strength of a single anchor

Vcpg  :	Concrete pryout strength of a group of anchors

Vn  :	Shear strength

Vsa  :	Strength in shear of a single anchor or group of anchors as governed by the steel strength

c,N  :	Factor used to modify tensile strength of anchors based on presence or absence of cracks in concrete

c,P  :	Factor used to modify pullout strength of anchors based on presence or absence of cracks in concrete

cp,N  :	Factor used to modify tensile strength of postinstalled anchors intended for use in uncracked concrete without supplementary reinforcement

ec,N  :	Factor used to modify tensile strength of anchors based on eccentricity of applied loads

ec,Nx  :	Factor used to modify tensile strength of anchors based on eccentricity in x axis of applied loads

ec,Ny  :	Factor used to modify tensile strength of anchors based on eccentricity in y axis of applied loads

ed,N  :	Factor used to modify tensile strength of anchors based on proximity to edges of concrete member

smin  :	Center-to-center anchor minimum spacing

SideFaceBlowoutApply  :	Side-face blowout apply

TensionShearInteraction  :	Result from tension-shear interaction

Vcp  :	Nominal pryout strength of a anchor in shear

Vcpg  :	Nominal pryout strength of a group of anchor in shear

Vua  :	Factored shear force applied to anchor or group of anchors

REFERENCES

[9] AISC 2005, Design Examples Version 13.0, pp. IIC-26 - IIC-27