| Section element | Rotation angle | Mirror | Material | E (kN/mm^2) |
| --- | --- | --- | --- | --- |
| Sheet 400 x 15 |  |  | Steel | 205.0 |
| Sheet 185 x 5 | 90.0 |  | Steel | 205.0 |
| Sheet 185 x 5 | 90.0 |  | Steel | 205.0 |
| Sheet 190 x 24 |  |  | Steel | 205.0 |

The overall dimensions of the section are 400 x 200 mm

Basic geometry of the section

|  | Parameter | Value |  |
| --- | --- | --- | --- |
| A | Cross sectional area | 12410.0 | mm2 |
|  | Angle between Y-Z and U-V axes | -90.0 | deg |
| Iy | Moment of inertia about axis parallel to Y passing through centroid | 90334116.12 | mm4 |
| Iz | Moment of inertia about axis parallel to Z passing through centroid | 111308416.67 | 4 |
| It | Torsional moment of inertia (St. Venant) | 1242984.42 | mm4 |
| iy | Radius of gyration about axis parallel to Y passing through centroid | 85.32 | mm |
| iz | Radius of gyration about axis parallel to Z passing through centroid | 94.71 | mm |
| Wu+ | Elastic modulus about U-axis (+ve extreme) | 556542.08 | mm3 |
| Wu- | Elastic modulus about U-axis (-ve extreme) | 556542.08 | mm3 |
| Wv+ | Elastic modulus about V-axis (+ve extreme) | 1020176.44 | mm3 |
| Wv- | Elastic modulus about V-axis (-ve extreme) | 810517.04 | mm3 |
| Wpl,u | Plastic modulus about axis parallel to U-axis | 996975.0 | mm3 |
| Wpl,v | Plastic modulus about axis parallel to V-axis | 999150.31 | mm3 |
| Iu | Moment of inertia about U-axis | 111308416.67 | 4 |
| Iv | Moment of inertia about V-axis | 90334116.12 | mm4 |
| iu | Radius of gyration about U-axis | 94.71 | mm |
| iv | Radius of gyration about V-axis | 85.32 | mm |
| au+ | Centroid to edge of compression zone along +ve U-axis | 82.21 | mm |
| au- | Centroid to edge of compression zone along -ve U-axis | 65.31 | mm |
| av+ | Centroid to edge of compression zone along +ve V-axis | 44.85 | mm |
| av- | Centroid to edge of compression zone along -ve V-axis | 44.85 | mm |
| yM | Distance to centroid along Y-axis | 200.0 | mm |
| zM | Distance to centroid along Z-axis | 88.55 | mm |
| yP | Distance to equal area axis along Y-axis | 200.0 | mm |
| zP | Distance to equal area axis along Z-axis | 141.06 | mm |
| Iyz | Moment of inertia Iyz in the user coordinates | 6.35275e-10 | mm4 |
| uP | Distance to equal area axis along U-axis | 52.52 | mm |
| vP | Distance to equal area axis along V-axis | 0.0 | mm |

File: D:\Admin Building\SECTION WIZARD\THQ 200X400.sec